Product Guide Specification

March 1, 2024

SECTION 02630

24-INCH CATCH BASINS

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	24-inch catch basins.

1.2	RELATED SECTIONS

A.	Section 02_______ - Excavation and Backfill.

B.	Section 02_______ - Drain Pipe.

1.3	SUBMITTALS

A.	Comply with Section 01330 - Submittal Procedures.

B.	Product Data:  Submit manufacturer's product data, including installation instructions.

C.	Shop Drawings:  Submit manufacturer's shop drawings, indicating materials, components, and dimensions.

D.	Warranty:  Submit manufacturer's standard warranty.

1.4	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area in accordance with manufacturer's instructions.

C.	Handling:  Protect materials during handling and installation to prevent damage.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	NDS, Inc., 851 North Harvard Avenue, PO Box 339, Lindsay, California 93247.  Toll Free (800) 726-1994.  Toll Free Fax (800) 726-1998.  Phone (559) 562-9888.  Fax (559) 562-4488.  Website www.ndspro.com.

2.2	24-INCH CATCH BASIN SERIES

A.	24-Inch by 24-Inch Square Grate:

1.	Part Number:  [2411]  [2412].

2.	Description:  24-inch square, structural-foam polyolefin, grate with UV inhibitor.

3.	Grate Openings:  7/8 inch.

4.	Open Surface Area:  231.69 square inches.

5.	Capacity:  710 gpm with ½ inch of head.

6.	Load:  61 to 175 psi.

7.	Color:  [Black]  [Green].

B.	24-Inch by 24-Inch Square Cast Iron Grate:

1.	Part Number:  2413.

2.	Description:  24-inch square, heavy-duty, cast iron, grate with black powder coating.

3.	Grate Openings:  1 inch.

4.	Open Surface Area:  197.00 square inches.

5.	Capacity:  604 gpm with ½ inch of head.

6.	Load:  176 to 325 psi.  H-20 rated.

7.	Color:  Black.

C.	24-Inch by 24-Inch Square Galvanized Steel Grate:

1.	Part Number:  2415.

2.	Description:  24-inch square, heavy-duty, galvanized steel, bar grate.

3.	Grate Openings:  1-1/4 inches.

4.	Open Surface Area:  422.45 square inches.

5.	Capacity:  1,294 gpm with ½ inch of head.

6.	Load:  176 to 325 psi.  H-20 rated.

7.	Color:  Galvanized steel.

D.	24-Inch by 24-Inch Catch Basin:

1.	Part Number:  [2400]  [2404].

2.	Description:  24-inch square, 1-piece, tapered catch basin.

3.	Openings:  [2]  [4].

4.	Color:  Black.

E.	24-Inch by 24-Inch Catch Basin Extension, No Bottom:

1.	Part Number:  2418.

2.	Description:  8-inch extension for 24-inch by 24-inch catch basin.

3.	Color:  Black.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive catch basins.  Notify Architect if areas are not acceptable.  Do not begin installation until unacceptable conditions have been corrected.

3.2	INSTALLATION

A.	Install catch basins in accordance with manufacturer's instructions at locations indicated on the drawings.

B.	Install catch basins and grates level and to proper elevation.

C.	Recess grates 1/8-inch below finish grade.

D.	Asphalt or Hot Mastic Applications:  Encase catch basins with 4-inch collar of concrete.

E.	Light Vehicular Traffic:  Install catch basins on top of compacted gravel, sand, or concrete and encase with 4-inch collar of concrete as indicated on the drawings.

F.	Install and tape grates before placing concrete or backfill.

3.3	PROTECTION

A.	Protect installed catch basins and grates from damage during construction.

END OF SECTION